CONCISE DESCRIPTION OF RELEVANCE: THIRD-PARTY SUBMISSION UNDER 37 CFR § 1.290

U.S. Application No. 19/318,450 (Publication No. US 2026/0070232 A1)

Document 7: JP 2007301640 A (“Document 7”)

First Named Inventor: Takeshi Ide

Publication Date: November 22, 2007

Title: Faceplate for magnetic chuck

This document discloses a faceplate for a magnetic chuck that releasably attracts a magnetic material such as a workpiece, featuring first and second pole shoes and spacers, and a magnetic flux generation source that can be switched between an excited and non-excited state. Its potential relevance to the claims of the above-identified application is set forth below.

| Claim language (US 2026/0070232 A1) | Relevant disclosure of this document |
| --- | --- |
| Claim 1: “A magnetic gripper for gripping a ferromagnetic workpiece, said magnetic gripper comprising” | The reference discloses a magnetic chuck that releasably attracts a magnetic material such as a workpiece. “The present invention relates to a face plate for a magnetic chuck that releasably attracts a magnetic material such as a workpiece.” • Paragraph [0004] |
| Claim 1: “a magnet which is arranged in the housing and which is displaceable along a displacement direction between a gripping position for gripping the ferromagnetic workpiece and a release position for releasing the ferromagnetic workpiece” | The reference discloses a magnetic flux generation source including an electromagnet device with an iron core and an exciting coil, and a permanent magnet device with fixed and movable magnets. “The magnetic flux generation source includes an electromagnet device including an iron core and an exciting coil wound around the iron core, a permanent magnet device including a plurality of fixed permanent magnets and a plurality of movable magnets, and …” • Paragraph [0071] |
| Claim 1: “a number of pole shoes attached to the housing, wherein each pole shoe has a workpiece contact surface and is designed to guide a magnetic field portion of the magnet to the workpiece contact surface” | The first and second magnetic pole pieces 20 and 22 and the spacer 24 jointly form a work surface for attracting the magnetic material. “The first and second magnetic pole pieces 20 and 22 and the spacer 24 jointly form a work surface for attracting the magnetic material on the side opposite to the main body 12.” • Paragraph [0090] |
| Claim 2 (The magnetic gripper according to claim 1, wherein the housing extends along a longitudinal axis) | The housing 18 is manufactured in the shape of a narrow box that is long in the longitudinal direction (first direction) of the main body 12. “The housing 18 is manufactured in the shape of a narrow box that is long in the longitudinal direction (first direction) of the main body 12.” • Paragraph [0061] |
| Claim 8 (The magnetic gripper according to claim 1, wherein the number of pole shoes includes a first pole shoe and a second pole shoe) | The faceplate includes a plurality of first pole shoes and a plurality of second pole shoes. “This faceplate includes a plurality of first pole shoes disposed at some interval apart in a first direction and being long in a second direction crossing with the first direction, a plurality of second pole shoes each disposed between the …” • Abstract • Paragraph [0164] |
| Claim 9 (The magnetic gripper according to claim 1, wherein the magnet is displaceable electrically, pneumatically, or mechanically between the gripping position and the release position) | When the supply of current to the exciting coil 20 is stopped, the work surface is brought into a non-excited state, and the attracted magnetic body is released. “When the supply of current to the exciting coil 20 is stopped, the work surface is brought into a non-excited state, and the attracted magnetic body is released.” • Paragraph [0168] |
| Claim 14 (wherein, in a release state, the magnet is in the release position and the ferromagnetic workpiece is not pressed, by means of a magnetic force, against each workpiece contact …) | When the supply of current to the exciting coil 20 is stopped, the work surface is brought into a non-excited state, and the attracted magnetic body is released. “When the supply of current to the exciting coil 20 is stopped, the work surface is brought into a non-excited state, and the attracted magnetic body is released.” • Paragraph [0168] • Paragraph [0004] |
| Claim 16 (The magnetic gripper according to claim 1, wherein the receptacle is in the form of a groove) | The face plate 14 is provided with a plurality of first grooves having a predetermined depth dimension. “In the example shown in the drawing, the face plate 14 is provided with a plurality of first grooves having a predetermined depth dimension and a constant pitch from the upper surface side of the plate body having a predetermined …” • Paragraph [0113] |